The Preventive Maintenance Program

Policy

A preventive maintenance program must be performed in order to guard against the breakdown of mechanical equipment, which can be expensive, inconvenient and time-consuming, for residents and for the owner.  A preventive maintenance program is designed to keep equipment operating at peak efficiency with minimum cost.  It:

- Extends the life of the Community’s mechanical systems.

- Increases the reliability of the equipment.

- Eliminates costly and labor-intensive repairs.

- Reduces energy consumption.

- Cuts down on spare parts inventories.

Preventive maintenance also improves the Community’s operating efficiency. Equipment that is beginning to fail should be identified and repaired at a time when its absence will not affect the Community or its residents.

EXAMPLE:  Servicing air conditioning chillers or pool pumps during the off season.

The preventive maintenance program will cover:

- Equipment that is essential to the Community’s operations, such as boilers, HVAC equipment, vents, motors, pumps, water heaters and swimming pool equipment.

- Building components that are subject to environmental or traffic deterioration, such as roofing, flashing, structural elements, paving and building interiors.

The preventive maintenance program calls for scheduled inspections and repair of the Community’s equipment and buildings. The program requires careful and detailed planning, implementation and follow-through.

# Procedure

The Preventive Maintenance Schedule Chart (see example) must be posted in the Manager’s Office or other area designated by the Manager.  This will provide the Regional Supervisor and Manager immediate updated information on the progress of the program.  The Regional Supervisor will designate region-specific weekly tasks on the Preventive Maintenance Schedule that are highlighted for identification.  A description of each task will be posted below the Preventive Maintenance Schedule Chart (see example). The Maintenance Technician will be responsible for insuring the completion of each weekly task in accordance with the schedules and standards set forth.

Once weekly tasks have been completed, the Maintenance Technician will update the Preventive Maintenance Schedule with the completed projects.  If a scheduled task is not completed in the designated time frame, the Maintenance Technician must provide the Supervisor with a written explanation.  The Property Manager and the Maintenance Technician will meet to determine if the project should be completed during the next cycle.  Only the Regional Supervisor can change the schedule of completion for items listed on the Preventive Maintenance Schedule.  Any additional inspection forms will be provided by the Regional Supervisor.

Only technically qualified maintenance associates with a working knowledge of the property’s operating systems to be serviced may perform the preventive maintenance procedures.

PREVENTIVE MAINTENANCE:  DESCRIPTION, DETAIL AND FREQUENCY

| # ITEM | ## DESCRIPTION | ## FREQUENCY |
| --- | --- | --- |
| LIGHTING, EXTERIOR | Street, parking, breezeway, signage, building and pathway. | Weekly |
| HVAC FILTERS | Replace filters and add algaecide tablets to one twelfth of the property weekly. Notify the units to be done the next week. Divide number of units on property by 12, this will give you the number of apts to be serviced weekly. EXAMPLE: 400 units divided by 12 equals 33.33 or 34 units per week. 200 units divided by 12 equals 17 units per week. | 4 times per year. |
| EVAPORATOR COILS | Coils are inspected during the second quarter (April, May, June) filter change. List coils for cleaning on a separate service request. If over 50% of the coils need cleaning, contact your RMD for further instructions. | Yearly. |
| SMOKE DETECTORS | Test according to local ordinance otherwise, quarterly. Any defective devices are to be replaced immediately. | 4 times per year. |
| PLUMBING, INTERIOR | Inspect all kitchen and bathroom faucets and toilets for leaks. Inspect all supply lines and washing machine hoses. List all repairs on service requests and schedule. Plumbing inspections and repairs are to be done during the first quarter (January, February, March) filter change. | Yearly. |
| DRAINS, STORM | Inspect and clear all property, storm, yard, compactor, courtyard, and/or downspout drains. | 4 times per year. |
| FURNACE | Inspect electrical and/or gas connections. Inspect heat exchangers and clean. Inspect burners and pilot flame. To be done in conjunction with Electrical inspection/ service in August, September, and October filter change. | Yearly. |
| ASPHALT AND SPEED BUMPS | Inspect and repair all streets, parking areas, entrances. Paint all speed bumps. | 3 times per year. |
| CURBING AND TIRE STOPS | Inspect and repair all curbing. Replace all broken or missing stops. Re-paint any previously painted curbing. | 3 times per year. |
| SIDEWALKS AND CONCRETE STEPS | Inspect all sidewalks and concrete steps. Mark and measure for possible capital, if necessary. | 4 times per year. |
| ELECTRICAL | Inspect breaker box. Tighten all breakers and main lugs. Inspect stove and dryer receptacles, bath exhaust fans, wall receptacles, etc. To be done in conjunction with Furnace service and the August, September, and October filter change. (Divide # units by 12 for weekly required unit #. 400 units divided by 12 is 34 units.) | Yearly. |
| FITNESS EQUIPMENT | Inspect and repair. | 4 times per year. |
| PLAYGROUND EQUIPMENT | Inspect all swings, slides, monkey bars and ground surface for safety and repair. | 6 times per year. |
| TENNIS AND BASKETBALL COURTS | Inspect and repair fencing, gates, wind screens, nets, backboards and posts. | 6 times per year. |
| CONDENSATION DRAINS | Clear all condensation drains through water or air pressure or through vacuuming drain lines. To be done in conjunction with the HVAC CONDENSER UNIT SERVICE. | Yearly. |
| SIGNAGE | Inspect for paint and plumb. Inspect for lighting. | Monthly. |
| BUILDING EXTERIORS | Use Exterior Inspection Form. | 4 times per year. |
| OFFICE, CLUBHOUSE, LAUNDRIES | Complete detailed interior and exterior punch list. Inspect paint, carpet, lighting, appliances, equipment, HVAC filters, venting, plumbing, roofing, guttering, windows, screens, alarm systems, rest rooms, and all other contents and structures. | Monthly. |
| BOILER SYSTEMS CIRCULATION PUMPS | Flush tanks, Inspect electrical, gas fittings, venting, burners, thermo-couples, temperature settings, motors (oiled), and thermostats or timers. Include EQR-owned laundry and clubhouse equipment. | 6 times per year. |
| COMPACTOR DUMPSTERS | Inspect steps/stairs/handrail/ramps. Inspect paint, equipment, switches, drains, concrete and dumpster enclosures. | Monthly. |
| POOLS, SPAS, SAUNAS | Use Inspection Form provided by the local county authority responsible for public pool safety. Replace motor/ pump seals yearly. Plumbing, filters, heaters, chlorinators, lighting, timers, motors and pumps are to be properly serviced. All recording forms must be current and chemicals properly stored. | Seasonally. |
| GOLF CARTS TRUCKS | Inspect brakes, batteries, windows, fuel system, seats and tires and clean thoroughly and detail. Perform all factory-advised servicing. Repair or replace as necessary. | 6 times per year. |
| FIRE ALARMS , SPRINKLER SYSTEMS | Inspect pull stations, alarms, relay stations, pump systems, and other equipment according to local ordinance. | 6 times per year. |
| GUTTERS, ROOFS, DOWNSPOUTS | Inspect guttering, flashing, shingles, fireplaces and caps, downspouts and splash blocks. Clean all gutters and roofs. Divide number of bldgs. by 7 to determine number of buildings to be cleaned each week. | Twice per year. |
| IRRIGATION SYSTEMS | Test all zones, valve boxes, timers, heads and clocks. Inspect and service motors and pumps. Repair as needed. | Yearly. |
| FENCING AND GATES | Inspect and repair all property fencing, including patios, pools and tennis courts. Inspect all gates and gate latches. | 6 times per year. |
| FOUNTAINS AND LAKES | Inspect pumps and lighting. Check for algae. | Monthly. |
| STAIRS AND BALCONIES | Inspect all treads, handrails, pickets and support structures. | 4 times per year. |
| WINDOWS AND DOORS | Inspect or replace all screens. Record all fogged windows/doors for capital. | Twice yearly. |
| HVAC CONDENSER UNITS | Inspect and clean units. Replace service port caps. Oil motors. Spray contactor. Inspect disconnect box. Inspect electrical. To be done in conjunction with Condensation Drain Service. Divide # units by 8 for weekly schedule. | Yearly. |